2015-16-11 11:37:17 SUP_V571A REL_V6.85F

-DPC system configuration-

[Tool: BD04] [Tube2: ASPOLYHK]

-------------------------------------------------------------------------

| RECIPES                                                                 |

|-------------------------------------------------------------------------|

| Number of recipes       : 4                                             |

| Number of abortrecipes  : 4                                             |

-------------------------------------------------------------------------

-------------------------------------------------------------------------

| PRESSURE CONTROL  (Internal Control)                                    |

|-------------------------------------------------------------------------|

|              Range                  |Proportional| Integral | Derivative|

|-------------------------------------|   factor   |  factor  |   factor  |

|  From  |  To   |     Units          |            |  [sec]   |   [sec]   |

|--------|-------|--------------------|------------|----------|-----------|

|    0   |  100  |     0 -  2000 mTor |     150    |   0.4    |     1.0   |

|-------------------------------------------------------------------------|

| Use improved leak rate calculation :  No                                |

-------------------------------------------------------------------------

-------------------------------------------------------------------------

| BOAT RANGE                                                              |

|-------------------------------------------------------------------------|

| Minimum position        :   10   [mm]                                   |

| Maximum position        : 1300   [mm]                                   |

| Boat rotation           :    3   [rpm]                                  |

| Number of pulses/minute :14400  (Read-Only data from BTC)               |

-------------------------------------------------------------------------

2015-16-11 11:37:17 SUP_V571A REL_V6.85F

-DPC digital input configuration-

[Tool: BD04] [Tube2: ASPOLYHK]

------------------------------------------------------------

| DIGITAL INPUTS                                             |

|------------------------------------------------------------|

| NR | NAME     | ON-TEXT  | OFF-TEXT | DESCRIPTION          |

|----|----------|----------|----------|----------------------|

|  1 | N2-Prs   | OK       | OUTRANGE | 2.5BarA <N2< 5BarA   |

|  2 | CA-Press | OK       | TOO LOW  | Press CA>4 Bar       |

|  3 | ExhGasc  | OK       | NO FLOW  | Airfl GasCab>1.5m/s  |

|  4 | ReacPrs  | OK       | >2TORR   | ReacPrs>2Torr        |

|  5 | PumpPrs  | OK       | >10TORR  | PumpPrs>10Torr       |

|  6 | -------- | -------- | -------- | -------------------- |

|  7 | ProcIntk | OK       | PROCINTK | Process Interlock    |

|  8 | CabDoors | CLOSED   | OPEN     | Cabinet Doors        |

|  9 | FlngTmp  | OK       | TOO LOW  | Flange Temp Too Low  |

| 10 | ExhReact | OK       | NO FLOW  | Furnace Cab.>1.5m/s  |

| 11 | TubeDoor | CLOSED   | OPEN     | Tube Door Cl Sensor  |

| 12 | OverHeat | OK       | OVERHEAT | Top Element >90+TC   |

| 13 | H2O-Reac | OK       | TOO LOW  | Waterflow Elem+Flan  |

| 14 | -------- | -------- | -------- | -------------------- |

| 15 | -------- | -------- | -------- | -------------------- |

| 16 | Tube-ATM | ATM      | VACUUM   | Atm switch Reac>1ATM |

| 17 | FacWarn  | OK       | WARNING  | Facility Warning     |

| 18 | PmpAlarm | OK       | PMPALM   | Pump Alarm           |

| 19 | -------- | -------- | -------- | -------------------- |

| 20 | MaintMod | AUTOMATC | MAINTENC | Maintenance Mode     |

| 21 | CtrlRem  | AUTOMATC | LOCAL    | Control Remote       |

| 22 | -------- | -------- | -------- | -------------------- |

| 23 | -------- | -------- | -------- | -------------------- |

| 24 | SiH4-Dep | SAFE     | UNSAFE   | SiH4 Deposition      |

| 25 | ExhBubbl | OK       | TOO LOW  | Exh. Bubbler >1.5m/s |

| 26 | -------- | -------- | -------- | -------------------- |

| 27 | FpmpSens | OPEN     | CLOSED   | Fastpump vlv Sensor  |

| 28 | -------- | -------- | -------- | -------------------- |

| 29 | ElevIntk | NOT ACTV | ACTIVE   | Elevator Interlock   |

| 30 | FacReady | OK       | ALARM    | Facility Ready       |

| 31 | GasLeak  | OK       | ALARM    | Gas Leak Detectors   |

| 32 | LeakDet  | OK       | ALARM    | HeatExch/TCU-Leak    |

------------------------------------------------------------

2015-16-11 11:37:17 SUP_V571A REL_V6.85F

-DPC digital output configuration-

[Tool: BD04] [Tube2: ASPOLYHK]

------------------------------------------------------------

| DIGITAL OUTPUTS                                            |

|------------------------------------------------------------|

| NR | NAME     | ON-TEXT  | OFF-TEXT | DESCRIPTION          |

|----|----------|----------|----------|----------------------|

|  1 | SiH4-E   | OPEN     | CLOSED   | SiH4-E valve         |

|  2 | SiH4-L   | OPEN     | CLOSED   | SiH4-L valve         |

|  3 | -------- | -------- | -------- | -------------------- |

|  4 | AsH3-D   | OPEN     | CLOSED   | AsH3-D valve         |

|  5 | -------- | -------- | -------- | -------------------- |

|  6 | AsH3-U   | OPEN     | CLOSED   | AsH3-U valve         |

|  7 | -------- | -------- | -------- | -------------------- |

|  8 | N2-L-M   | OPEN     | CLOSED   | N2-L manifold valve  |

|  9 | N2-D     | OPEN     | CLOSED   | N2-D valve           |

| 10 | N2-L     | OPEN     | CLOSED   | N2-L valve           |

| 11 | N2-U     | OPEN     | CLOSED   | N2-U valve           |

| 12 | SiH4-E-M | OPEN     | CLOSED   | SiH4-E manifold vlv  |

| 13 | SiH4-L-M | OPEN     | CLOSED   | SiH4-L manifold vlv  |

| 14 | N2-E     | OPEN     | CLOSED   | N2-E valve           |

| 15 | -------- | -------- | -------- | -------------------- |

| 16 | -------- | -------- | -------- | -------------------- |

| 17 | N2-U-M   | OPEN     | CLOSED   | N2-U manifold valve  |

| 18 | FastPump | OPEN     | CLOSED   | Fast pump valve      |

| 19 | SlowPump | OPEN     | CLOSED   | Slow pump valve      |

| 20 | SiH4-Evc | OPEN     | CLOSED   | SiH4 evac valve      |

| 21 | AsH3-Evc | OPEN     | CLOSED   | AsH3 evac valve      |

| 22 | SiH4-Src | OPEN     | CLOSED   | SiH4 source valve    |

| 23 | AsH3-Src | OPEN     | CLOSED   | AsH3 source valve    |

| 24 | -------- | -------- | -------- | -------------------- |

| 25 | SiH4-Prg | OPEN     | CLOSED   | SiH4 purge valve     |

| 26 | AsH3-Prg | OPEN     | CLOSED   | AsH3 purge valve     |

| 27 | AsH3-D-M | OPEN     | CLOSED   | AsH3-D manifold vlv  |

| 28 | N2-E-M   | OPEN     | CLOSED   | N2-E manifold valve  |

| 29 | N2-D-M   | OPEN     | CLOSED   | N2-D manifold valve  |

| 30 | AsH3-U-M | OPEN     | CLOSED   | AsH3-U manifold vlv  |

| 31 | -------- | -------- | -------- | -------------------- |

| 32 | -------- | -------- | -------- | -------------------- |

------------------------------------------------------------

2015-16-11 11:37:17 SUP_V571A REL_V6.85F

-DPC analog input configuration-

[Tool: BD04] [Tube2: ASPOLYHK]

-----------------------------------------------------

| ANALOG INPUTS                                       |

|-----------------------------------------------------|

| NR | LABEL    | RANGE | UNIT | DESCRIPTION          |

|----|----------|-------|------|----------------------|

|  1 | -------- | ----- | ---- | -------------------- |

|  2 | SiH4-Prs | 125.0 | PsiA | SiH4 Pressure        |

|  3 | -------- | ----- | ---- | -------------------- |

|  4 | N2PrsFw  | 10.00 | SLM  | N2 Purge Flow        |

|  5 | LeakRate | 200.0 | mT/m | Leak Rate            |

|  6 | Leak-in  | 50.0  | mTor | Leak In Level        |

|  7 | N2PrgCar | 300   | SLM  | N2 Carrousel Purge   |

|  8 | AsH3-Prs | 125.0 | PsiA | Ash3 Pressure        |

|  9 | N2-Prs   | 125.0 | PsiA | N2 Pressure          |

| 10 | -------- | ----- | ---- | -------------------- |

| 11 | ATM-Pres | 1000  | Torr | ATM Pressure         |

| 12 | TmpFlng  | 200.0 | °C   | Flange Temp          |

| 13 | ATM-Setp | 1000  | Torr | ATM SetPoint         |

| 14 | -------- | ----- | ---- | -------------------- |

| 15 | FlngLiqT | 200.0 | °C   | Flange Liquid Temp.  |

| 16 | -------- | ----- | ---- | -------------------- |

-----------------------------------------------------

2015-16-11 11:37:17 SUP_V571A REL_V6.85F

-DPC analog output configuration-

[Tool: BD04] [Tube2: ASPOLYHK]

----------------------------------------------------------------------

| ANALOG OUTPUTS                                                       |

|----------------------------------------------------------------------|

| NR | LABEL    | INTERLOCK | RANGE | UNIT | RAMP| DESCRIPTION         |

|----|----------|-----------|-------|------|-----|---------------------|

|  1 | SiH4-E   | CLOSED    | 1.000 | SLM  | 8   | SiH4-E              |

|  2 | SiH4-L   | CLOSED    | 1.000 | SLM  | 8   | SiH4-L              |

|  3 | -------- | --------- | ----- | ---- | --- | --------------------|

|  4 | AsH3-D   | CLOSED    | 200.0 | SCCM | 8   | AsH3-D              |

|  5 | -------- | --------- | ----- | ---- | --- | --------------------|

|  6 | AsH3-U   | CLOSED    | 200.0 | SCCM | 8   | AsH3-U              |

|  7 | -------- | --------- | ----- | ---- | --- | --------------------|

|  8 | TubePres | PRESSURE  | 2000  | mTor | 0   | Tube Press Control  |

|  9 | N2-D     | CLOSED    | 5.00  | SLM  | 8   | N2-D                |

| 10 | N2-L     | CLOSED    | 100.0 | SCCM | 8   | N2-L                |

| 11 | N2-U     | CLOSED    | 100.0 | SCCM | 8   | N2-U                |

| 12 | O2-Level | NO        | 1000  | PPM  | 0   | O2 Concentration    |

| 13 | -------- | --------- | ----- | ---- | --- | --------------------|

| 14 | N2-E     | CLOSED    | 5.00  | SLM  | 8   | N2-E                |

| 15 | -------- | --------- | ----- | ---- | --- | --------------------|

| 16 | -------- | --------- | ----- | ---- | --- | --------------------|

----------------------------------------------------------------------

2015-16-11 11:37:17 SUP_V571A REL_V6.85F

-DPC message configuration-

[Tool: BD04] [Tube2: ASPOLYHK]

---------------------------

| MESSAGES                  |

|---------------------------|

| NR | MESSAGE              |

|----|----------------------|

|  1 | STANDBY              |

|  2 | BOAT IN/OUT          |

|  3 | STABILIZE            |

|  4 | TIME DELAY           |

|  5 | N2-PURGE             |

|  6 | PRE-DEP.             |

|  7 | DEPOSITION           |

|  8 | POST-DEP.            |

|  9 | SLOW PUMP            |

| 10 | FAST PUMP            |

| 11 | BACKFILL             |

| 12 | LEAKCHECK            |

| 13 | EVAC                 |

| 14 | PROFILE              |

| 15 | ISOLATE              |

| 16 | ABORTED!!            |

---------------------------