2015-16-11 11:37:05 SUP_V571A REL_V6.85F

-DPC system configuration-

[Tool: BD04] [Tube1: TIN1]

-------------------------------------------------------------------------

| RECIPES                                                                 |

|-------------------------------------------------------------------------|

| Number of recipes       : 4                                             |

| Number of abortrecipes  : 4                                             |

-------------------------------------------------------------------------

-------------------------------------------------------------------------

| PRESSURE CONTROL  (External Control)                                    |

|-------------------------------------------------------------------------|

| Use improved leak rate calculation : Yes                                |

-------------------------------------------------------------------------

-------------------------------------------------------------------------

| BOAT RANGE                                                              |

|-------------------------------------------------------------------------|

| Minimum position        :   10   [mm]                                   |

| Maximum position        : 1300   [mm]                                   |

| Boat rotation           :    3   [rpm]                                  |

| Number of pulses/minute :14400  (Read-Only data from BTC)               |

-------------------------------------------------------------------------

2015-16-11 11:37:05 SUP_V571A REL_V6.85F

-DPC digital input configuration-

[Tool: BD04] [Tube1: TIN1]

------------------------------------------------------------

| DIGITAL INPUTS                                             |

|------------------------------------------------------------|

| NR | NAME     | ON-TEXT  | OFF-TEXT | DESCRIPTION          |

|----|----------|----------|----------|----------------------|

|  1 | N2-Press | OK       | OUTRANGE | 2.5BarA <N2< 5BarA   |

|  2 | CA-Press | OK       | TOOLOW   | Press CA>4BarG       |

|  3 | ExhGasC  | OK       | NOFLOW   | Airfl GasCab>1.5m/s  |

|  4 | ReacPrs  | OK       | >2TORR   | Reac Prs>2 Torr      |

|  5 | PumpPres | OK       | >10TORR  | Pump Prs>10 Torr     |

|  6 | FRBlower | ON       | OFF      | FR Blower Status     |

|  7 | ProcIntk | OK       | PROCINTK | Process Interlock    |

|  8 | CabDoors | CLOSED   | OPEN     | Cabinet Doors        |

|  9 | Flngalrm | OK       | ALARM    | Flange Temp          |

| 10 | ExhReact | OK       | NOFLOW   | Furnace Cab>1.5m/s   |

| 11 | TubeDoor | CLOSED   | OPEN     | Tube Door Cl Sensor  |

| 12 | OverHeat | OK       | OVERHEAT | Top Element >90+TC   |

| 13 | H2O-Reac | OK       | TOOLOW   | Waterflow Elem+Flan  |

| 14 | DoorTemp | OK       | ALARM    | Heated Door Low Temp |

| 15 | 2ndPmpPr | OK       | >100TORR | 2nd PumpPr >100 Torr |

| 16 | Tube-ATM | ATM      | VACUUM   | Atm switch Reac>1ATM |

| 17 | FacWarn  | OK       | WARNING  | Facility Warning     |

| 18 | PmpAlarm | PUMPON   | PMPALM   | Pump Alarm           |

| 19 | VacVlvT  | OK       | ALARM    | Vacuum Valve Temp    |

| 20 | MaintMod | AUTOMATC | MAINTENC | Maintenance Mode     |

| 21 | CtrlRem  | AUTOMATC | LOCAL    | Control Remote       |

| 22 | FRH2OTmp | OK       | TOOHIGH  | FR H2O Temp Alarm    |

| 23 | 2ndPmpAl | PUMPON   | PMPALM   | 2nd Pump Alarm       |

| 24 | GenReady | READY    | NOTREADY | Gen Stream Ready     |

| 25 | ExhBubbl | OK       | TOOLOW   | Exh. Bubbler >1.5m/s |

| 26 | FRAirTmp | OK       | TOOHIGH  | FR Air Temperature   |

| 27 | SmokeDet | OK       | ALARM    | Smoke Detection      |

| 28 | TraceTmp | OK       | OUTBOUND | Heating Alarm        |

| 29 | ElevIntk | NOTACTV  | ACTIVE   | Elevator Interlock   |

| 30 | FacReady | OK       | ALARM    | Facility Ready       |

| 31 | GasLeak  | OK       | ALARM    | Gas Leak Detectors   |

| 32 | DPAlarm  | OK       | ALARM    | Diff PT Status       |

------------------------------------------------------------

2015-16-11 11:37:05 SUP_V571A REL_V6.85F

-DPC digital output configuration-

[Tool: BD04] [Tube1: TIN1]

------------------------------------------------------------

| DIGITAL OUTPUTS                                            |

|------------------------------------------------------------|

| NR | NAME     | ON-TEXT  | OFF-TEXT | DESCRIPTION          |

|----|----------|----------|----------|----------------------|

|  1 | N2-Prg   | OPEN     | CLOSED   | Bar Tree Prg Vlv     |

|  2 | N2-U     | OPEN     | CLOSED   | N2-U Vlv             |

|  3 | N2-L     | OPEN     | CLOSED   | N2-L Vlv             |

|  4 | N2-D-L   | OPEN     | CLOSED   | N2-D Low Vlv         |

|  5 | N2-D-H   | OPEN     | CLOSED   | N2-D High Vlv        |

|  6 | NF3      | OPEN     | CLOSED   | NF3 Vlv              |

|  7 | SiH4     | OPEN     | CLOSED   | SiH4 Vlv             |

|  8 | HeatDoor | ACTIVE   | NOTACTV  | Heated Door Plate    |

|  9 | NH3-H    | OPEN     | CLOSED   | NH3 High Vlv         |

| 10 | N2-TiCl  | OPEN     | CLOSED   | N2-TiCl4 Vlv         |

| 11 | O2       | OPEN     | CLOSED   | O2 Vlv               |

| 12 | NH3-Prg  | OPEN     | CLOSED   | NH3 Purge Vlv        |

| 13 | N2-Misc  | OPEN     | CLOSED   | N2-C/U/L/D Mn Vlv    |

| 14 | Cl2      | OPEN     | CLOSED   | Cl2 Vlv              |

| 15 | FRUpStr  | OPEN     | CLOSED   | FastRamp Upstream    |

| 16 | FRDnStr  | OPEN     | CLOSED   | FastRamp Downstream  |

| 17 | BypasPmp | OPEN     | CLOSED   | Bypass Pump Vlv      |

| 18 | FastPump | ON       | OFF      | Fast Pump Mode       |

| 19 | SlowPump | ON       | OFF      | Slow Pump Mode       |

| 20 | NF3-Prg  | OPEN     | CLOSED   | NF3 Purge Vlv        |

| 21 | NH3-Src  | OPEN     | CLOSED   | NH3 Source Vlv       |

| 22 | SiH4-Prg | OPEN     | CLOSED   | SiH4 Purge Vlv       |

| 23 | TiCl4Prg | OPEN     | CLOSED   | TiCl4 Purge Vlv      |

| 24 | TiCl4    | OPEN     | CLOSED   | TiCl4 Vlv            |

| 25 | TiCl4Evc | OPEN     | CLOSED   | TiCl4 Evac Vlv       |

| 26 | TiCl4-Mn | REACTOR  | PUMP     | TiCl4 Mn Vlv         |

| 27 | NF3-Mn   | REACTOR  | PUMP     | NF3 Mn Vlv           |

| 28 | O2-Mn    | REACTOR  | PUMP     | O2 Mn Vlv            |

| 29 | Cl2-Mn   | REACTOR  | PUMP     | Cl2 Mn Vlv           |

| 30 | Cl2-Prg  | OPEN     | CLOSED   | Cl2 Purge Vlv        |

| 31 | NH3-Mn   | REACTOR  | PUMP     | NH3 Mn Vlv           |

| 32 | SiH4-Mn  | REACTOR  | PUMP     | SiH4 Mn Vlv          |

------------------------------------------------------------

2015-16-11 11:37:05 SUP_V571A REL_V6.85F

-DPC analog input configuration-

[Tool: BD04] [Tube1: TIN1]

-----------------------------------------------------

| ANALOG INPUTS                                       |

|-----------------------------------------------------|

| NR | LABEL    | RANGE | UNIT | DESCRIPTION          |

|----|----------|-------|------|----------------------|

|  1 | VacVlv1  | 200.0 | °C   | Vacuum Vlv Temp 1    |

|  2 | VacVlv2  | 200.0 | °C   | Vacuum Vlv Temp 2    |

|  3 | NH3-Prs  | 100.0 | PsiA | NH3 Pressure         |

|  4 | Vlv-Pos  | 100.0 | %    | Vacuum Vlv Position  |

|  5 | LeakRate | 200.0 | mT/m | Leak Rate (only DPC) |

|  6 | Leak-in  | 50.0  | mTor | Leak Input Level     |

|  7 | N2PrgCar | 300   | SLM  | N2 Car Purge         |

|  8 | Cl2-Prs  | 100.0 | PsiA | Cl2 Pressure         |

|  9 | N2-Prs   | 100.0 | PsiA | N2 Pressure          |

| 10 | HeatDoor | 230   | °C   | Heated Door Temp     |

| 11 | ATM-Prs  | 1000  | Torr | ATM Pressure         |

| 12 | NF3-Prs  | 100.0 | PsiA | NF3 Pressure         |

| 13 | SiH4-Prs | 100.0 | PsiA | SiH4 Pressure        |

| 14 | TiCl-Prs | 250   | PsiA | TiCl4 Pressure       |

| 15 | BotFlng  | 200.0 | °C   | Bottom Flange Temp   |

| 16 | O2-Prs   | 100.0 | PsiA | O2 Pressure          |

-----------------------------------------------------

2015-16-11 11:37:05 SUP_V571A REL_V6.85F

-DPC analog output configuration-

[Tool: BD04] [Tube1: TIN1]

----------------------------------------------------------------------

| ANALOG OUTPUTS                                                       |

|----------------------------------------------------------------------|

| NR | LABEL    | INTERLOCK | RANGE | UNIT | RAMP| DESCRIPTION         |

|----|----------|-----------|-------|------|-----|---------------------|

|  1 | N2-Prg   | CLOSED    | 10.00 | SCCM | 1   | Bar Tree Prg Flow   |

|  2 | N2-U     | CLOSED    | 1.000 | SLM  | 1   | N2-U Flow           |

|  3 | N2-L     | CLOSED    | 1.000 | SLM  | 1   | N2-L Flow           |

|  4 | N2-D-L   | CLOSED    | 200.0 | SCCM | 1   | N2-D Low Flow       |

|  5 | N2-D-H   | CLOSED    | 20.00 | SLM  | 1   | N2-D High Flow      |

|  6 | NF3      | CLOSED    | 200.0 | SCCM | 1   | NF3 Flow            |

|  7 | SiH4     | CLOSED    | 1.000 | SLM  | 1   | SiH4 Flow           |

|  8 | TubePres | PRESSURE  | 2000  | mTor | 0   | Tube Pressure Ctrl  |

|  9 | NH3-H    | CLOSED    | 20.00 | SLM  | 1   | NH3 High Flow       |

| 10 | N2-TiCl  | CLOSED    | 500   | SCCM | 1   | N2-TiCl4 Flow       |

| 11 | O2       | CLOSED    | 1.000 | SLM  | 1   | O2 Flow             |

| 12 | O2-Contr | NO        | 1000  | PPM  | 0   | O2 Concentration    |

| 13 | FRBlower | NO        | 100.0 | %    | 0   | FR Blower Speed     |

| 14 | Cl2      | CLOSED    | 200.0 | SCCM | 1   | Cl2 Flow            |

| 15 | TiCl     | NO        | 1.600 | g/m  | 0   | TiCl4 Flow          |

| 16 | NH3-L    | NO        | 2.000 | SLM  | 1   | NH3 Low Flow        |

----------------------------------------------------------------------

2015-16-11 11:37:05 SUP_V571A REL_V6.85F

-DPC message configuration-

[Tool: BD04] [Tube1: TIN1]

---------------------------

| MESSAGES                  |

|---------------------------|

| NR | MESSAGE              |

|----|----------------------|

|  1 | STANDBY              |

|  2 | BOAT IN/OUT          |

|  3 | STABILIZE            |

|  4 | TIME DELAY           |

|  5 | N2-PURGE             |

|  6 | PRE-DEP.             |

|  7 | DEPOSITION           |

|  8 | POST-DEP.            |

|  9 | SLOW PUMP            |

| 10 | FAST PUMP            |

| 11 | BACKFILL             |

| 12 | LEAKCHECK            |

| 13 | EVAC                 |

| 14 | PROFILE              |

| 15 | ISOLATE              |

| 16 | ABORTED!!            |

---------------------------