AKRION Automated WET Bench

Configuration:

Tank 1:

Recirc GAMA Module (SPM)

Quartz Re-circulated Process Tank

Sulfuric (H2SO4)

Chemical Ratio: 9:1

Single HDPE Gravity Dispense Vessel for H2O2 (Using LE100 Level Sensors)

Process Temperature: 90 - 150°C ± 2.0°C

In-line Heater

Bubbler Level Sensors

10:1 Dilution Drain

Tank 2:

Quick Dump Rinse Module (QDR)

Tank 3:

SC1 PLUS Megasonic Module (Single Use)

Series 3 Quartz Megasonic Process Tank

Automatic Chemical Blending for SC1 ratios (50:2:1,100:2:1 and NH4OH only)

In-line Heater - Process temperature – 40 – 60° C

Tank 4:

SC1 PLUS Megasonic Module (Single Use)

Series 3 Quartz Megasonic Process Tank

Automatic Chemical Blending for SC1 ratios (50:2:1, 100:2:1 & NH4OH only)

In-line Heater - Process temperature – 40 – 60° C

Dryer:

IPA Dryer Module

PVDF Tank Assembly with Auto Lid

Bulk Fill IPA