Inspection Report

Maker	: TEL

Model	: Alpha-303iK

Serial Number	: L000004YXXXX

- . Issue Item Summary

| NO | Module Name | Status |  | Remark |
| --- | --- | --- | --- | --- |
| General Configuration |  | Available | None |  |
| 1 | Process | Oxide |  |  |
| 2 | Wafer Size | 300mm |  |  |
| 3 | Process Wafer Qty | 100 Pcs |  |  |
| 4 | Boat Quantity | 2 Pics |  |  |
| 5 | I/O Port / SMIF | Foup Type |  |  |
| 6 | N2 Load Lock | YES |  |  |
| 7 | Carrier Slot | 25 Slots |  |  |
| System Control |  | Available ○ | None X |  |
| 1 | System Controller Model | ECC Rack |  |  |
| 2 | Operation Panel | Touch Panel |  |  |
| 3 | Gas Flow Chart (Front & Rear) | Front | Rear |  |
|  |  | ○ | ○ |  |
| 4 | Temperature Controller Model | M560 |  |  |
| 5 | Controller Transformer | 1Ø 120V |  |  |
| 6 | Heater Transformer | 3Ø 440V, 50 KVA |  |  |
| 7 | Oxygen Analyzer | X |  |  |
| 8 | HEC Pyro Ctl (HCT) | ○ |  |  |

| NO | Module Name | Status |  | Remark |
| --- | --- | --- | --- | --- |
| Process Gasses |  | Available ○ | None X |  |
| 1 | Process Gas 1 | N2 |  |  |
| 2 | Process Gas 2 | O2 |  |  |
| 3 | Process Gas 3 | H2 |  |  |
| 4 | Process Gas 4 | HCl |  |  |
| 5 | Process Gas 5 | — |  |  |
| 6 | Process Gas 6 | — |  |  |
| 7 | Process Gas 7 | — |  |  |
| Process Liquid |  | Available ○ | None X |  |
| 1 | Process Liquid 1 | — |  |  |
| 2 | Process Liquid 2 | — |  |  |
| 3 | Process Liquid 3 | — |  |  |
| Gas Distribution System |  | Available ○ | None X |  |
| 1 | Basic Style | IGS Type |  |  |
| 2 | Tubing Material | SUS 316L-LM |  |  |
| 3 | Tubing Finish | VCR Type |  |  |
| 4 | Manual Valve | Fujikin |  |  |
| 5 | Air-Operated Valve | Fujikin |  |  |
| Exhaust Distribution System |  | Available ○ | None X |  |
| 1 | Air-Operated Valve Type | Fugikin |  |  |
| 2 | Main Valve | None |  |  |
| 3 | Cold Trap | ○ |  |  |
| 4 | Pump Line | ○ |  |  |
| 5 | Butterfly Valve(manual mode) | ○ |  |  |
| Wafer/Cassette Handling |  | Available ○ | None X |  |
| 1 | Wafer Alignment |  | X |  |
| 2 | Cassettle Storage Qty | 16 |  |  |
| 3 | Cassette In/Out Port | ○ |  |  |
| 4 | cassette Handling Robot | ○ |  |  |
| 5 | Wafer Transfer Type | TEL |  |  |
| 6 | Fork Material | Ceramic |  |  |
| 7 | Fork Qty | 4+1 |  | Currently there are only the second and third |
| 8 | Fork Variable Pitch | ○ |  |  |
| Elevator Handling |  | Available ○ | None X |  |
| 1 | Boat Elevator | ○ |  |  |
| 2 | Auto Shutter | ○ |  |  |
| 3 | Boat Rotation |  | X |  |
| Heating Chamber |  | Available ○ | None X |  |
| 1 | Heater Model | VMM-56-002 |  |  |
| 2 | Heater Zone | 5 |  |  |
| 3 | RCU |  | X |  |
| 4 | RCU Pipe(From heater to RCU) |  | X |  |