标准配置

1 机器人本体

2 焊锡控制器（UP-05）

3 教导器（RK-TP）

4 烙铁头清洗装置（UJC-214）

5 焊头（UMC-087）

6 锡丝供给器（UPM-011A）

7 锡丝供给导管（PT系列）

8 操作盒（RK-SWB）

Standard configuration:
1. Robot body
2. Soldering controller (UP-05)
3. Teaching pendant (RK-TP)
4. Soldering tip cleaning device (UJC-214)
5. Soldering tip (UMC-087)
6. Solder wire feeder (UPM-011A)
7. Solder wire feeding tube (PT series)
8. Operation box (RK-SWB)

规格（机器人本体）

Specifications (Robot Body)

| 可移动轴数 Number of movable axes |  | 4（同步控制） 4 (Synchronous Control) |
| --- | --- | --- |
| 可移动范围 Range of movement | X-轴 X-axis | 300mm |
|  | Y-轴 Y-axis | 320mm |
|  | Z-轴 Z-axis | 100mm |
|  | R-轴 R-axis | ± 360° |
| 最大可搬动重量（工件） Maximum movable weight (workpiece) |  | 11Kg |
| 最大移动速度（PTP） Maximum moving speed (PTP) | X，Y-轴 X，Y -axis | 800mm/sec.(8-800mm/sec.) |
|  | Z-轴 Z-axis | 320mm/sec.(3.2-320mm/sec.) |
|  | R-轴 R-axis | 800°/sec.(8-800°/sec.) |
| 重复精度 Repeatability accuracy | X，Y-轴 X，Y -axis | ±0.01mm |
|  | Z-轴 Z-axis | ±0.01mm |
|  | R-轴 R-axis | ±0.02° |
| 工件尺寸（最大夹具尺寸） Workpiece size (maximum fixture size) |  | 300mm×275mm(X,Y) |
| X轴台面，烙铁头与台面之间的距离（Z轴方向） X-axis work surface, the distance between the soldering iron tip and the work surface (in the Z-axis direction) |  | 在焊头未UM-087时：145mm When the welding head is not UM-087: 145mm |
| 位置教导方法 Position teaching method |  | 直接移动各轴教导位置模式（JOG）或位置坐标数据直接输入模式（MDI） Directly move each axis in the teach position mode (JOG) or directly input position coordinate data in the MDI mode. |
| 教导方式 Teaching methods |  | 通过教导器直接教导 用PC电脑在off-line状态下页可以教导（可选备件） Teaching can be done directly through the teaching device. Teaching can also be conducted offline using a PC (optional accessory). |
| 程序数据 Program data |  | 255个程序 255 programs |
| 焊点最大储存数量 Maximum storage quantity of solder joints |  | 30000个焊点 30,000 solder joints |
| 驱动系统 Drive system |  | 5-相马达驱动 Five-phase motor drive |
| 控制系统 control system |  | PTP控制，CP控制 PTP control, CP control |
| 辅助计算功能 Auxiliary calculation function |  | 3次元直线辅助计算，圆弧辅助计算功能 3D straight line auxiliary calculation, arc auxiliary calculation function |
| 外部接口 External interface |  | RS232接口2个（1个可外接PC；另一个可外接选备件） RS422接口一个（用于接教导器） Two RS232 interfaces (one for external connection to a PC; the other for optional accessories) One RS422 interface (for connection to a teaching pendant) |
| 外部输入输出（max.) External input/output (max.) |  | 输入24点，输出24点 Input 24 points, output 24 points. |
| 简易PLC功能 Simple PLC functions |  | 50个程序，每个程序内可以写入1000步命令 There are 50 programs, and each program can have up to 1000 command steps written into it. |
| 外观尺寸 Appearance dimensions |  | W：615×D：581×H861（mm) |
| 电源Power supply |  | AC90-120V 50/60Hz 450VA |
| 压缩空气 Compressed air |  | 高燥空气 4-5Kg/cm2 High-dry air 4-5 Kg/cm² |
| 相关湿度 Relative humidity |  | 0-40℃ |
| 相关湿度 Relative humidity |  | 20-95% |
| 重量 weight |  | 39kg |

规格 焊锡控制器 Specification: Soldering Controller

| 型号 Model | UP-05 |
| --- | --- |
| 焊锡条件的数量 The number of soldering conditions | 63 |
| 烙铁头温度的可调节范围 The adjustable range of the soldering iron tip temperature | 最大450摄氏度 Up to 450 degrees Celsius |
| 加热器报警值Heater alarm value | ±0-100℃可变范围 Variable range: ±0 - 100℃ |
| 加热器功率 Heater power | 200W加热器*2 Two 200W heaters |
| 锡丝直径范围 The diameter range of tin wire | 0.5-1.2（指定） 0.5 - 1.2 (as specified) |
| 锡丝供给控制器 Tin wire supply controller | stepping motor高精度控制 High-precision control of stepping motor |
| 报警 Alarm | 无锡。锡丝堵塞。加热器报警 No tin. Tin wire clogging. Heater alarm. |
| 工作环境 working environment | 0-40℃ |
| 相关湿度 Relative humidity | 20-95%（无霜） 20 - 95% (frost-free) |
| 重量 weight | 22Kg |